Lcd.c

#include <LPC21xx.h>

#include "mlcd.h"

#include "Timer.h"

#include <LPC21xx.h>

#include "mlcd.h"

#include "Timer.h"

#define RS      (1<<8)      // P0.8

#define EN      (1<<9)      // P0.9

#define LCDDATA (0xF<<4)    // P0.4 - P0.7  // P0.4 - P0.7

static void lcd_enable(void)

{

IOSET0 = EN;

delay_ms(2);

IOCLR0 = EN;

delay_ms(2);

}

void lcd_cmd(unsigned char cmd)

{

IOCLR0 = RS;

IOCLR0 = LCDDATA;

IOSET0 = (cmd & 0xF0);

lcd_enable();

IOCLR0 = LCDDATA;

IOSET0 = ((cmd << 4) & 0xF0);

lcd_enable();

}

void lcd_data(unsigned char data)

{

IOSET0 = RS;

IOCLR0 = LCDDATA;

IOSET0 = (data & 0xF0);

lcd_enable();

IOCLR0 = LCDDATA;

IOSET0 = ((data << 4) & 0xF0);

lcd_enable();

}

void LCD_INIT(void)

{

IODIR0 |= RS | EN | LCDDATA;

delay_ms(20);

lcd_cmd(0x02);   // 4-bit mode

lcd_cmd(0x28);   // 2 lines, 5x7 font

lcd_cmd(0x0C);   // Display ON

lcd_cmd(0x06);   // Cursor increment

lcd_cmd(0x01);   // Clear display

delay_ms(5);

}

void lcd_string(char *str)

{

while(*str)

{

lcd_data(*str++);

}

}

void lcd_num(unsigned int num)

{

char buf[10];

int i = 0;

if(num == 0)

{

lcd_data('0');

return;

}

while(num > 0)

{

buf[i++] = (num % 10) + '0';

num /= 10;

}

while(i > 0)

{

lcd_data(buf[--i]);

}

}

Mlcd.h

#ifndef __MLCD_H__

#define __MLCD_H__

#include <LPC21xx.h>

void LCD_INIT(void);

void lcd_cmd(unsigned char cmd);

void lcd_data(unsigned char data);

void lcd_string(char *str);

void lcd_num(unsigned int num);

#endif

Timer.h

#ifndef __TIMER_H__

#define __TIMER_H__

#include <LPC21xx.h>

void delay_ms(unsigned int ms);

#endif

TIMER.C

#include <LPC21xx.h>

#include "Timer.h"

void delay_ms(unsigned int ms)

{

T0TCR = 0x02;          // Reset Timer0

T0PR  = 15000 - 1;     // 1 ms delay

T0TCR = 0x01;          // Start Timer0

while(ms--)

{

T0TC = 0;

while(T0TC < 1);

}

T0TCR = 0x00;          // Stop Timer0

}

Led.h

#ifndef __LED_H__

#define __LED_H__

#include <LPC21xx.h>

#define LED (1<<13)

void LED_INIT(void);

void LED_ON(void);

void LED_OFF(void);

void LED_BLINK(void);

#endif

Led.c

#include <LPC21xx.h>

#include "led.h"

#include "Timer.h"

void LED_INIT(void)

{

IODIR0 |= LED;      // P0.13 as Output

IOCLR0 = LED;       // LED OFF

}

void LED_ON(void)

{

IOSET0 = LED;

}

void LED_OFF(void)

{

IOCLR0 = LED;

}

void LED_BLINK(void)

{

LED_ON();

delay_ms(200);

LED_OFF();

}

Switch.h

#ifndef __SWITCH_H__

#define __SWITCH_H__

#include <LPC21xx.h>

#define SW (1<<12)

void SWITCH_INIT(void);

unsigned char switch_READ(void);

#endif

Switch. C

#include <LPC21xx.h>

#include "switch.h"

void SWITCH_INIT(void)

{

IODIR0 &= ~SW;      // P0.12 as Input

}

unsigned char switch_READ(void)

{

if((IOPIN0 & SW) == 0)

return 1;       // Switch Pressed

return 0;           // Switch Released

}

Interrupt. h

#ifndef __INTERRUPT_H__

#define __INTERRUPT_H__

#include <LPC21xx.h>

extern volatile unsigned int inside_count;

extern volatile unsigned int total_in;

extern volatile unsigned int total_out;

void interrupt_init(void);

#endif

Interrupt. C

#include <LPC21xx.h>

#include "interrupt.h"

#include "led.h"

unsigned int inside_count = 0;

unsigned int total_in = 0;

unsigned int total_out = 0;

/* IR1 - ENTRY */

void EINT0_ISR(void) __irq

{

inside_count++;

total_in++;

LED_ON();

for(int i = 0; i < 50000; i++);

LED_OFF();

EXTINT = (1<<0);

VICVectAddr = 0;

}

/* IR2 - EXIT */

void EINT1_ISR(void) __irq

{

if(inside_count > 0)

{

inside_count--;

total_out++;

}

LED_ON();

for(int i = 0; i < 50000; i++);

LED_OFF();

EXTINT = (1<<1);

VICVectAddr = 0;

}

void interrupt_init(void)

{

PINSEL0 |= (1<<29);

PINSEL1 |= (1<<0);

EXTMODE = 0x03;

EXTPOLAR = 0x00;

VICIntSelect = 0x00;

VICVectAddr0 = (unsigned)EINT0_ISR;

VICVectCntl0 = 0x20 | 14;

VICVectAddr1 = (unsigned)EINT1_ISR;

VICVectCntl1 = 0x20 | 15;

VICIntEnable = (1<<14) | (1<<15);

}

Main.c

#include <LPC21xx.h>

#include "mlcd.h"

#include "Timer.h"

#include "led.h"

#include "switch.h"

#include "interrupt.h"

int main(void)

{

LCD_INIT();

LED_INIT();

SWITCH_INIT();

interrupt_init();

while(1)

{

/* Normal Display */

lcd_cmd(0x01);

lcd_cmd(0x80);

lcd_string("VISITOR COUNT");

lcd_cmd(0xC0);

lcd_string("IN:");

lcd_num(inside_count);

lcd_string(" OUT:");

lcd_num(total_out);

delay_ms(300);

/* Switch Press - Show Total Counts */

if(switch_READ())

{

lcd_cmd(0x01);

lcd_cmd(0x80);

lcd_string("TOTAL IN");

lcd_cmd(0xC0);

lcd_num(total_in);

delay_ms(2000);

lcd_cmd(0x01);

lcd_cmd(0x80);

lcd_string("TOTAL OUT");

lcd_cmd(0xC0);

lcd_num(total_out);

delay_ms(2000);

while(switch_READ());

}

}

}